Adaptive Equalization Using the LMS Algorithm

- An adaptive equalizer is an equalizer that automatically adapts to time-varying properties of the communication channel. It is frequently used with coherent modulations such as phase shift keying, mitigating the effects of multipath propagation and Doppler spreading.

CODE:

% Adaptive equalization

% Simulate some (useful) signal to be transmitted

>> ni=6000;	% length of input

a=(randn(1,ni)>0)*4-1;	% Random bipolar

>> nn=length(a);

% Channel Model

%w=2.89;

w=3;

>>  h = [0,0.5*(1 + cos(2*(pi/w)*(-1:1)))];

>>  n = length(h);

%ah=conv(h,a);

>>  ah=filter(h,1,a);

mu=0.012;

>> plot(a,'.')

>> plot(ah,'.')

>>  NN=11;	%adaptive filter length

>> w=zeros(1,NN);	%initial adaptive filter coefficient

v=sqrt(0.001)*randn(1,ni);	%Gaussian noise with variance 0.001;

u=ah(1:ni)+v;

delay=NN-1;

px=zeros(1,NN);

%for 1=NN+1:ni

%y(i)=w’*u((i-NN):(i-1));

%e(i)=a(i-delay)-y(i);

%w=w+mu*e(i)*u((i-NN):(i-1));

%J(i)=e(i)^2;

%end

E=[];aa=[zeros(1,delay) a(1:ni-delay)];

>>  for i=1:ni;

px=[u(i) px(1:length(px)-1)];

y(i)=w*px';

e=aa(i)-y(i);

w=w+mu*e*px;

end

pause

subplot(221), plot(a,'.'), title('original signal to be transmitted')

subplot(222), plot(ah,'.'), title('convolved (distorted)signal h*a')

subplot(223), plot(u,'.'), title('signal after addition of noise')

subplot(224),plot(y, '.'),title('Output signal')

for i=1:length(y);

if y(i)>=0.5,S(i)=1;

else S(i)=-1;

end

end

>>  tr=a(1:length(a)-(NN-1));

rx=S(NN:length(S));

k=0;

for j=1:length(tr);

if tr(j)~=rx(j), k=k+1;

end

end

BER=k./(length(rx));

pause

subplot(221), stem((h)), title('channel input response h(n)')

subplot(222), stem(ah,'.'), title('convolved (distorted)signal h*a')

CONCLUSION:

By performing this code we learnt the equalizing properties of an adaptive equalizer. It automatically adapts the time varying properties of a communicating channel. We can get the value of output as we want by changing the value of “mu”.